Aston Technologies Inc.

Cisco Security Lab -Windows Services and Windows Hosts

An Aston training document stepping through deploying our hosts needed for the Cisco security labs

Contents

Contents	2

Introduction	3

Deploy Windows Machines	4

Configure Windows Machines	9

Conclusion	18

Introduction

This lab we are going to deploy 4 Windows machines. A Windows 2016 server with Active Directory, Certificate Authority and DNS services already installed. Then we will deploy 3 Windows 10 hosts that we will use to test our policies in the upcoming labs.

Ideally you should use the vcenter management interface at https://dc-isevcen.astontech.com/ to operate everything in the lab but if you have a requirement to access any of the lab devices directly from Astons internal network, you can use the IP table below.

|  | LAB 1 | LAB 2 | LAB 3 | LAB 4 |
| --- | --- | --- | --- | --- |
| ACCESS-SW (MGMT) | 10.8.1.35 | 10.8.1.40 | 10.8.1.42 | 10.8.1.44 |
| CORE-SW (MGMT) | 10.8.1.36 | 10.8.1.41 | 10.8.1.43 | 10.8.1.45 |
| ASA (outside) | 10.8.1.2 | 10.8.1.66 | 10.8.1.98 | 10.8.1.130 |
| Jumphost (RDP) | 10.8.1.5 | 10.8.1.69 | 10.8.1.101 | 10.8.1.133 |
| FTD (outside) | 10.8.1.10 (P) 10.8.1.11 (S) | 10.8.1.75 (P) 10.8.1.76 (S) | 10.8.1.105 (P) 10.8.1.106 (S) | 10.8.1.135 (P) 10.8.1.136 (S) |

Deploy Windows Machines

Open the vCenter management interface at https://dc-isevcen.astontech.com/ and sign in with your Aston AD credentials. Contact a mentor or escalation engineer if permissions are required.

Once signed in click the icon in the image below bring up the VMs and Templates view. Expand the templates folder for the lab you are working in. There is a folder for lab 1 and a folder for lab 2-4. Right Click on ISE-AD SERVER TEMPLATE and select New VM Machine from this Template.

Name it YOUR-NAME AD SERVER and put it in the corresponding VM folder. Then hit Next.

VMs for lab 1 go in esximnise-1.astontech.com and VMs for the rest go in esximnise-2.astontech.com then hit Next.

Put it in one of the VM-datastores 1 or 2 which ever has the most space and hit Next. Labs 2-4 will have only one datastore option.

Select Customize this virtual machine’s hardware and hit Next.

Change Network adapter 1 to the MGMT subnet in the corresponding lab number.

Then hit Next and Finish.

Now right click on the ISE-PC-TEMPLATE and select New VM from this Template. Name it YOUR Name PC-1 and put it in the LAB X VMs folder. Then hit Next.

Select the proper ESXi host(1 for lab 1 and 2 for the rest)  and hit Next

Put it in one of the VM-datastores 1 or 2 whichever has the most space and hit Next.

Select Customize this virtual machine’s hardware and hit Next.

Assign Network adapter 1 to the MGMT SUBNET of the proper lab pod then hit Next and Finish.

Now create two more PCs named YOUR NAME PC-2 and YOUR NAME PC-3 follow the same process we just completed.

Configure Windows Machines

Once the Windows Server is been deployed open a console window and log in with Administrator qwe123$!. Active Directory, DNS, CA and GPO has already been configured for you. Create a user for yourself. Make your user a Member Of the following groups – BYOD-Users, Domain Admins, Employee, Network Admins and Sponsor.

Now open a console window to PC-1. Log in with admin qwe123$!. Hit the Windows icon and type PC. Then Right click and select Properties.

Hit Change Settings.

Hit Change.

Name it PC-1.

Restart the VM.

Do the same thing for PC-2 and PC-3.

Once that is complete, open the console window to PC-1 again. Click on the Windows icon and type PC. Then right click on it and go to Properties.

Add the PC to the domain. Under Computer name, domain and workgroup settings hit Change settings.

Hit the Change button on the next window.

Hit the Domain radio button and type ise.astontech.com in the window then hit OK.

Use the Administrator account and password qwe123$! to join the domain.

You should see the Welcome message. Hit OK, Ok then Close.

Then Restart Now.

Now PC-1 should be joined to the domain. Do the same thing for PC-2 but not for PC-3.

After the reboot sign in with your AD account for PC-1 and PC-2. Click Other User then sign in.

Let’s check to see if the Certs were issued. Open the console window for the 2016 server. Open the Start Menu and click on Windows Administrative tools.

Then click Certification Authority then expand ise-AD-DC-CA and click on Issued Certificates. You should see a cert for PC-1 and PC-2 and two for yourself.

Before we wrap up this lab move PC-1 from the MGMT network to LAB (x) Access-SW G1/0/8.

Conclusion

In this lab, we have:

- Deployed a Windows 2016 server from a template

- Deployed three Windows 10 hosts from a template

- Added ourselves a user in Active Directory and applied group membership

- Joined ISE-PC-1 and ISE-PC2 to the domain

- Verified that the certificates were issued from the CA

In the next lab, we are going to bootstrap our ISE host and make some tweaks to the initial configuration.