Aston Technologies Inc.

Cisco Identity Services Engine (ISE) Wireless BYOD Dual SSID

An Aston training document explaining how to deploy wireless BYOD with dual SSIDs

Contents

Contents	2

Lab Diagram	3

WLC Configuration	4

Add New SSID	4

Create ACL	6

ISE Configuration	8

Authorization Profiles	8

Policy Set	9

Onboarding BYOD Devices	10

Windows 10	10

Android Tablet	15

Apple iPad	21

Conclusion	28

Lab Diagram

WLC Configuration

Add New SSID

We are going to create an SSID that will be dual purposed as in we are going to use it for Guest access in a later lab as well. Log into the WLC and go to WLANs. Select Create New then hit Go.

Name it ISE-GUEST-(x) then hit Apply.

Hit Apply. Go to the Security tab and set the Layer2 Security to None. Check the MAC filtering checkbox.

Go to AAA Servers and add ISE for Authentication and Accounting Servers.

Go to the Advanced tab.

Check:

- Allow AAA Override

- DHCP Addr. Assignment Required

- Under Radius Client Profiling check:

  - DHCP Profiling

  - HTTP Profiling

- Change the NAC State to ISE NAC

Hit Apply and Save Configuration.

Create ACL

Go to Security > Access Control Lists and click New. Name it ASTON-BYOD-iOS and add the following entries:

Click into ASTON-BYOD-iOS and create the following rules:

Go to WLANs click into ISE-GUEST-(x) and in the General tab, check the Status Enabled checkbox.

At this point we can also we can stop broadcasting our ISE-CORP-(x) SSID.  Click into ISE-CORP-(x) and in the General tab, uncheck Broadcast SSID.

Then Save Configuration.

ISE Configuration

Authorization Profiles

Most of the ISE configuration is already in place from the previous BYOD labs. We only need to add a few Authorization Profiles then build our authC and authZ policies. Log in to ISE and go to Work Centers > BYOD > Policy Elements > Results > Authorization Profiles. Click Add and name it ASTON-WIRELESS-CWA.

In the Common Tasks section check Web Redirection. We want to do Centralized Web Auth, the redirect ACL is ISE-ACCESS that is configured on the WLC and the Value is the guest portal we set up in the wired BYOD lab – ASTON-CWA. Then Submit.

Duplicate the Profile we just created and name it ASTON-BYOD-iOS. All we need to change here is the redirect ACL to match the iOS one we created earlier on the WLC. Then Submit.

Policy Set

First, we need to add an Authentication policy for wireless MAB. Go to Policy Sets > WIRELESS and insert a rule above the Default. Name it MAB. Conditions we want to match is Wireless_MAB and Allowed Protocols will be MAB-HOST-LOOKUP. Then we need to change Identity Source to Internal Endpoints and set it to Continue If user not found.

We also need to add an Authorization policy to force the redirect to our CWA portal. Insert a rule above the Default. Name it CWA-REDIRECT. For the conditions we want to match on Wireless_MAB AND Normalised Radius:SSID CONTAINS ISE-GUEST-(x). Then for Permissions give it ASTON-WIRELESS-CWA.

Now we need to create a policy for our iOS devices for the mini browser (CNA). Duplicate above the on the CWA-REDIRECT rule we just created. Name it BYOD-DUAL-SSID-iOS. The Condition we want to add is Session:BYOD-Apple-MiniBrowser-Flow EQUALS Yes. Note: This is a new condition in ISE 2.2 for this particular use case. Then for Permissions give it ASTON-BYOD-iOS.

Save. That’s it. We already created the BYOD-REGISTERED rule that we will match once we get the device gets onboarded.

Onboarding BYOD Devices

Windows 10

We are ready to start to test out the dual SSID solution but first we are going to need to remove the credentials that we installed in the last lab. Console into LAB-PC-3. Click on your wireless adaptor in the task bar and click Network & Internet Settings.

Go to Wi-Fi and then click Manage known networks.

Click on ISE-CORP-(x) and hit Forget.

Open MMC and remove the certificate that was installed previously.

Log in to https://mydevices.lab.astontech.com/ and Delete all the devices.

Alright we are ready to test. On ISE-PC-3 connect to the ISE-GUEST-(x) SSID, a browser should automatically open and you’ll be redirected to the guest portal. Provide your credentials and hit Sign On.

Hit Start.

Provide a Device Name.

Save the Network Setup Assistant and then Run it.

Click Yes to continue.

Hit Start to configure the Network Settings.

Hit Yes to Delete the Root Certificate.

Hit Yes to install the Root Cert.

Boom! You should be connected to ISE-CORP-(x) now and have full network access.

You can see here we come in authenticating via MAB then after we get onboarded there at the top we are using our cert with EAP-TLS and get the full access profile.

Take a minute to review the ISE logs. Also, look at the client info in the WLC, at My Devices Portal, ISE CA issued certs and Registered devices. Make sure everything is as expected.

Android Tablet

For this part of the lab just follow along. This is just for you to see the process. We no longer have access to the wireless network for these labs.

Same as with the Windows host we need to remove the credentials on the Android Tablet from the previous lab. Go to Settings > Wi-Fi. Click on ISE-CORP and hit Forget.

Now Connect to ISE-GUEST.

Click CONTINUE ANYWAY VIA BROWSER.

Sign in to the portal.

Hit Start.

Enter a Device Name.

Now hit the home button then Click on the Network Setup Assistance icon.

Start the Network Setup Assistant.

Click Proceed.

Enter your AD password and hit OK.

Change Credential use to Wi-Fi and hit OK.

Enter 123456 and hit OK.

Change Credential use to Wi-Fi and hit OK.

The onboard process is complete. Hit Exit. You should now be connected to ISE-CORP.

Yay, we can get to cisco.com.

A quick look at the Live Logs and we see what we expect to see.

Take a minute to review the ISE logs. Also, look at the client info in the WLC, at My Devices Portal, ISE CA issued certs and Registered devices. Make sure everything is as expected.

Apple iPad

For this part of the lab just follow along. This is just for you to see the process. We no longer have access to the wireless network for these labs.

Before you begin and try to onboard the iPad have your mentor reset it for you. Since we aren’t using well known certificates the iPad seems to have an issue when you delete the profile and try to connect again.

Apple iOS 10.3.x has a few known issues with ISE BYOD onboarding that disrupt the flow which are supposed to be fixed with iOS 11. So for now, we are going to have to deal with it.

Go to Settings > Wi-Fi and connect to ISE-GUEST. Click Continue.

Sign in to the Portal.

When you get to this screen, just leave it alone and hit the home button. You should be able to hit Click here after you see ‘Done’ but this isn’t working with this code version.

Open Safari and go to 1.1.1.1.

You should get redirected to the Portal Page. Hit Start.

Enter a Device Name.

Click Launch Apple Profile and Certificate Installers Now button.

Hit Install to install the Root Cert.

Hit Install again.

Hit Done.

When you see this prompt for the second time go to Settings > General > About > Certificate Trust Settings and enable our Root Cert.

Hit Continue.

Double tap the Home button and go back to the previous page. Then hit Allow.

Hit Install.

Hit Install once more.

You should see it going through the Enrollment process at this point.

When it’s finished hit Done.

Go to Settings > Wi-Fi > ISE-GUEST and hit Forget this Network.

Now you automatically connect to ISE-CORP.

A quick look at the Live Logs and we see what we expect to see.

Conclusion

In this lab, we have:

- Configured an SSID for Guest on the WLC

- Added an ACL on the WLC for our Apple iPad use case

- Configured Authorization Profiles for wireless CWA and for iOS devices on ISE

- Created authentication policy for wireless MAB on ISE

- Created authorization policy for CWA redirect and iOS with dual SSIDs

- Onboarded and tested three different common BYOD device types:

  - Windows 10 host

  - Android tablet

  - Apple iPad

In the next lab, we are going to start working on our guest access flows.