Aston Technologies Inc.

Cisco Identity Services Engine (ISE) Remote Access VPN

An Aston training document explaining how to deploy Remote Access VPN

Contents

Contents	2

Lab Diagram	3

ASA Configuration	4

VPN Address Pool	4

NAT Configuration	5

Add AAA Server	6

Tunnel Group Configuration (AKA Connection Profiles)	7

Split Tunneling Policy	10

Group Policy	10

AnyConnect Client Profile	11

ISE Configuration	13

Downloadable ACLs	13

Authorization Profiles	14

Policy Set	15

Testing VPN Access	16

Conclusion	23

Lab Diagram

ASA Configuration

VPN Address Pool

For our users to have access to our internal network we need to give them an IP address that is routable internally. There are a number of ways to do this 1) configure the ASA to use our internal DHCP server 2) configure an address pool on the ASA for Remote Access users. There are more less prevalent ways to accomplish this but these two are the most common.  We are going to use number 2 which is probably the most commonly deployed method.

Navigate to Configuration > Remote Access VPN > Network (Client) Access > Address Assignment > Address Pools. Click Add and name it VPN_POOL and give it the following IP addressing:

Then hit Apply.

NAT Configuration

We need to create a NAT rule so our VPN users return traffic doesn’t get NAT’d.

Go to Configuration > Firewall > NAT Rules and hit Add. In the Original Packet section click on Destination Address box. Click Add to create a network object. Name it VPN_POOL_NAT0 and configure the following:

Configure the NAT rule as follows:

Hit OK and Apply.

Add AAA Server

Next, let’s add a server group for ISE like we did for TACACS except we are going to use RADIUS this time. Navigate to Configuration > Device Management > Users/AAA > AAA Server Groups. Click Add, give it a name of ISE and configure the following:

Now let’s add a server to the group. Click the Add button on the bottom section and configure the following (Secret Key – cisco123):

Hit Apply and test Authentication.

Tunnel Group Configuration (AKA Connection Profiles)

Navigate to Configuration > Remote Access VPN > Network (Client) Access > AnyConnect Connection Profiles and click the checkbox to Enable Cisco AnyConnect VPN Client Access. Hit Yes on the popup to designate an AnyConnect image.

Hit Browse Flash and select the anyconnect-win-4-x.xxxxx-webdeploy-k9-pkg image and hit OK.

Allow access on the outside interface as shown below:

We are going to use ISE to control what Group Policy our users get based on their Active Directory group membership. By default, if you don’t allow users to select a connection profile (tunnel group) upon login they will be directed to the DefaultWEBVPNGroup. Let’s edit that profile to fit our needs. Click on the DefaultWEBVPNGroup and select Edit. Configure the following on the Basic tab and hit OK then Apply:

Split Tunneling Policy

Split Tunneling allows you to specify what you want or don’t want to be sent through the tunnel. In our case, we only want to send traffic to our internal network through the tunnel and allow the user to browse the internet etc. locally.

We are going to create a Standard ACL to tell the ASA or the AnyConnect client what IP addresses to send through the tunnel. Navigate to Configuration > Firewall > Advanced > Standard ACL. Click Add ACL and name it SPLIT-TUNNEL-ACL. Then Add ACE and give it the address of 172.16.0.0/12. Hit Ok then Apply.

Group Policy

We are going to create one group policy and use ISE to allow for differentiated access to the network. Navigate to Configuration > Remote Access VPN > Network (Client) Access > Group Policies and click Add. Name it EMPLOYEE then click More Options, uncheck Inherit on the Tunneling Protocols and check SSL VPN Client. Everything else on this page can stay default.

Expand the Advanced tab and select Split Tunneling. For Policy, we want to Tunnel Network List Below and for the Network List we want to use the SPLIT-TUNNEL-ACL we created earlier.

Hit OK and Apply.

AnyConnect Client Profile

Last thing we’ll do on the ASA is create an AnyConnect Client Profile. Navigate to Configuration > Remote Access VPN > Network (Client) Access > AnyConnect Client Profile and click Add. Name it AnyConnect_VPN_PROFILE and apply our EMPLOYEE group policy. Then hit OK.

If you get a message saying it already exists hit Overwrite File.

Click on our newly created policy and hit Edit. Go to the Server List tab and hit Add. Configure your ASA outside IP address:

Then hit OK and Apply.

ISE Configuration

Downloadable ACLs

We already have added the ASA into ISE as a network device, so we are ready to start creating policy. Log into ISE and navigate to Work Centers > Network Access > Policy Elements > Results > Downloadable ACLs.

We are going to add ACLs for our Employees and for Network Admins. Click Add and name it ASTON-VPN-EMPLOYEE. For this one we are going to deny ICMP and permit everything else.

Add another one and name it ASTON-VPN-NETWORK-ADMIN. We are going to just do a permit any on this one.

Authorization Profiles

Now we need to create our authorization profiles for our two groups. We need to do two things here one is apply the dACL and the other is to send the class attribute equal to the group policy we want to apply to the user.

Go to Authorization Profiles and click Add. Name it ASTON-VPN-EMPLOYEE. In the Common Tasks check DACL Name and give it ASTON-VPN-EMPLOYEE.

Then Scroll down in the Common Tasks down to ASA VPN. We want to add our Group Policy exactly like it is configured on the ASA. Click the checkbox and input OU=EMPLOYEE. Then hit Save.

Add another name it ASTON-VPN-NETWORK-ADMIN. This is going to be the same except the ACL is going to be ASTON-VPN-NETWORK-ADMIN.

Policy Set

We are going to add a new Policy Set for VPN. Click Add and create a new Policy Set. Name it VPN and for the conditions we are going to match on DEVICE:Device Type EQUALS Device Type#All Device Types#Firewall AND Radius:NAS-Port-Type EQUALS Virtual.

For the Authentication Policy, we are going to just edit the default. Change the Identity Source to ASTON-AD.

Now we need to create our Authorization Policies. Create a new rule and name it NETWORK ADMIN. We want to match on ASTON-AD:ExternalGroups EQUALS Network Admins then give it ASTON-VPN-NETWORK-ADMIN for permissions.

Duplicate below and name it EMPLOYEE. Change the AD group to Employee and the permissions to ASTON-VPN-EMPLOYEE. Then Save.

Testing VPN Access

Log into vCenter, open a console window to LAB-PC-2 and log in. Change the Network Adaptor to LAB OUTSIDE/INTERNET.

We don’t currently have a DHCP server for this subnet. Statically Assign an IP appropriately as shown below -

| Gateway | Lab 1 | Lab 2 | Lab 3 | Lab 4 |
| --- | --- | --- | --- | --- |
| 10.8.1.225/27 | 10.8.1.226 – 228 | 10.8.1.229 – 232 | 10.8.1.233 – 236 | 10.8.1.237 – 240 |

Lab 4 example -

Open the AnyConnect client and connect to your VPN gateway. Hit Change Setting on the popup.

Uncheck the Block connections to untrusted servers.

Try your connection again. Click Connect Anyway on the popup.

Input your creds and hit OK.

Now you should be connected and have full access to the ISE lab. Try pinging some devices and going to ISE in a web browser.

If we check the Live Logs in ISE we passed, and the ACL was downloaded.

Dive into the details and take a closer look.

If we look from the ASA side in ASDM by going to Monitoring > VPN > VPN Statistics > Sessions then filtering on AnyConnect Client you can see we are getting the EMPLOYEE Group Policy.

Click the Details button you’ll get more information on the tunnel. If you hit the ACL tab you can see the ACL and ACEs that are applied to the session.

You can see the same information in the ASA CLI. Log into the ASA CLI and run the command show vpn-sessiondb detail anyconnect.

Same as with a switch you can see what’s in the access-list by doing a show access-list <name>.

Now let’s test the regular employee account. Log into LAB-AD-DC and add User1 to the Employee group.

Back to ISE-PC-2 and Disconnect the current VPN session. Reconnect and use the User1 account.

Try pinging our AD server or ISE. This should fail but you should be able to access the AD server or ISE via a web browser.

Check the Live Logs in ISE and the ASA and make sure everything is as expected.

Conclusion

In this lab, we have:

- Configured the ASA to support RA VPN with the AnyConnect client

- Added ISE to use as the backend AAA server for RA VPN

- Configured dACLs for each group in our use case in ISE

- Created Authorization Profiles for our RA VPN groups

- Configured a new Policy Set for VPN access in ISE

- Tested both types of accounts access to the internal network

In the next lab, we are going to go a step further and use certificates to authenticate to the VPN. We’ll use the ISE CA for our non-domain machines and our internal CA for our domain machines.