Aston Technologies Inc.

Cisco Identity Services Engine (ISE) Posture Assessment with AnyConnect

An Aston training document explaining how to deploy basic Posture with ISE AnyConnect Posture Module

Contents

Contents	2

Lab Diagram	3

ISE Configuration	4

Client Provisioning	4

Policy Elements	8

Posture Policy	15

Policy Set	15

Configure Access SW	16

Testing with ISE-PC-2	17

Conclusion	31

Lab Diagram

ISE Configuration

Client Provisioning

For posturing to work with ISE we need to utilize the AnyConnect ISE posture and compliance modules. There are two ways to get AnyConnect on the device. One way would be to use the enterprise’s corporate software distribution system but since we don’t have one we are going to set up ISE to push out the software. Along with Software we need to send a configuration profile like we did we the VPN labs. Let’s get started.

Log into ISE and navigate to Work Centers > Posture > Settings > Software Updates > Posture Updates and make sure that your posture information is up to date. You can hit Update Now to make sure.

Navigate to Work Centers > Posture > Client Provisioning > Resources. Hit Add and select AnyConnect Posture Profile.

Name it ASTON_ISE_AC_PROFILE. For the Agent behavior let’s give ourselves a little more time for the Remediation timer also Enable the Rescan Button. Configure the following:

Everything else can stay default except under the Posture Protocol we need to add a Server name rule. This tells the server what domain it’s allowed to connect to in the case of overlapping IP space. Add *.lab.astontech.com. Then hit Save.

Now we need to upload the AnyConnect software version that we are using to ISE. Hit Add and select Agent resources from local disk.

For Category select Cisco Provided Packages.

Browse to the AnyConnect version. If you don’t have it locally on your machine you can grab it from our shared drive \\10.1.249.115\Software\Networking\Data\Security\Cisco\ASA\AnyConnect\4.7. Then hit Submit.

Now hit Add again and select AnyConnect Configuration.

Select the AnyConnect Package. Name it ASTON AnyConnect Windows. Select the latest Compliance Module.

Add the Profile we created under ISE Posture. Everything else can stay default. Hit Save.

Next go to the Client Provisioning portal. Select the Create. Name it ASTON POSTURE and hit Save. Wait tell you see the Saved window popup before moving on.

Navigate to the Client Provisioning Policy. We could use the BYOD policies to push out the Posture software but we have those set up for BYOD-USERS and we want to posture everyone.

Add a new policy at the top of the list. Name it ASTON POSTURE WINDOWS. Operating System is going to be Windows All and for the let’s match on dot1x (Radius:Service-Type EQUALS Framed) and the Employee AD group then for Results under the Agent Configuration select ASTON AnyConnect Windows. Then hit Save.

Policy Elements

We are going to create 3 checks for posture. One is not really a check but rather collecting information on what applications are installed and running on the host. For the other two we are going to require that the Windows Firewall is enabled and Windows defender is installed and up to date.

Navigate to Policy Elements > Conditions > Application and hit Add. Name it ASTON-APP-COLLECTION-WINDOWS and configure the following then Save:

Select Firewall Condition and hit Add. Configure the following then hit Save:

Select Anti-Malware and hit Add. Name it ASTON-MALWARE-WINDOWS and configure the following and Save:

Now it’s on to Remediations. For Application since we are just collecting information they isn’t really anything to remediate and there is a default remediation for Anti-Malware we can use. We’ll need to create one for our Firewall check however.

Select Firewall and hit Add. Name it ASTON-FW-WINDOWS and configure the following then Save:

We have it set to manual remediation for now. Later we’ll change that to see how the behavior changes.

Select Requirements and Insert new requirement, it doesn’t matter where. These requirements aren’t processed in a top down order like you would see within the Policy Sets for example.

Name this requirement ASTON APP COLLECTION WIN. Operating System will be Windows All, Compliance module 4.x or later and conditions select ASTON-APP-COLLECTION-WINDOWS.

Duplicate the App requirement we just created and change the name to ASTON ANTI-MALWARE WIN. For conditions change to ASTON-MALWARE-WINDOWS. For the Remediation Actions select AnyAMDefRemediationWin.

Duplicate one of the new requirements and change the name to ASTON FW WINDOWS. Change the conditions to ASTON-FIREWALL-WINDOWS and then for Remediation Actions configure the following:

Then hit Save.

Now we need to create some dACLs. One for when the posture status is unkown and another for when the host is non-compliant. We’ll use the ASTON-EMPLOYEE ACL when the host is compliant.

Select Downloadable ACLs and hit Add. Name it ASTON-NON-COMP-POSTURE and configure the following:

Add another and name it ASTON-UNKNOWN-POSTURE and configure the following:

Now we need to make three Authorization Profiles one for each one of our posture states.

Select Authorization Profiles and hit Add. Name the first one ASTON-COMP-POSTURE-WIRED. All we need to do here is send the dACL.

Add another one and name it ASTON-NON-COMP-POSTURE-WIRED. For this profile, we are going to send a dACL and change the VLAN to the QUARNTINE VLAN we have configured on the switch. You can put the VLAN name or ID which is 35.

Last one name ASTON-UNKNOWN-POSTURE-WIRED. For this profile, we’ll need to set up web redirection the client provisioning portal as well as send a dACL. For the dACL select ASTON-UNKNOWN-POSTURE.

For the web redirection select Client Provisioning (Posture) and for the ACL type POSTURE-DISCOVERY which we will add to the switch a little later. Then select the posture portal that we created - ASTON POSTURE.

Hit Save.

Posture Policy

Navigate to Posture Policy and add a new policy. This is where we are going to say the who, what, when and where for posture.

Name the Policy ASTON EMPLOYEE WINDOWS. Operating system will be Windows All, Compliance Module – 4.x or later. Then for the conditions we want to match on our AD group Employee. Then we are going to apply the three requirements that we created.

Policy Set

We need to configure some rules in the WIRED policy set. Navigate to Policy Sets > WIRED. We are going to use ISE-PC-2 to do the testing so let’s create new rules with EAP-FAST.

Duplicate Above the EMPLOYEE EAP CHAIN policy. Name it EMPLOYEE COMPLIANT. We are going to add one condition which is Session:PostureStatus Equals Compliant.

Duplicate Above the EMPLOYEE COMPLIANT policy and name it EMPLOYEE NON-COMPLIANT. We are going to change the posture status to NOT EQUALS Compliant and change the results to ASTON-NON-COMP-POSTURE.

Duplicate Above EMPLOYEE NON-COMPLIANT and name it EMPLOYEE UNKNOWN POSTURE. Change the Posture Status to EQUALS Unknown. Then change the results to ASTON-UNKNOWN-POSTURE.

Then Save.

Configure Access SW

We need to configure redirect ACL for the posture discovery process. Log in to Access-SW and configure the following:

ip access-list extended POSTURE-DISCOVERY

permit tcp any 172.16.0.1 0.0.255.0 eq www

permit tcp any host 72.163.1.80 eq www

Testing with ISE-PC-2

Let’s get the PC ready for testing. Open a console window to PC-2 and log in. Remove the static IP that we added in the VPN lab.

Ok, now we should be ready to connect to the network to test. Change the Network adaptor to LAB (x) Access SW G1/0/10.

We need to install the ISE posture module; in the real world you would use your enterprise’s software distribution tool but since we don’t have one, we’ll need kick off the install manually.

Once you change the interface; open a web browser and type 172.16.11.1. That should redirect you to the provision portal. Once that page loads click on Start.

ISE is checking for AnyConnect ISE posture and compliance modules.

Click This is my first time here.

Click where it says, “Click here to download and install AnyConnect”.

Click Run.

Hit Yes.

Hit Connect.

You can see that AnyConnect is installing.

Now AnyConnect is scanning the system.

Once that’s finished scanning you should now be compliant and have access to the whole network.

Test access to the internet and Ping the Access-sw.

Now we want to fail posture. Let’s disable the Windows Firewall. Hit the Windows key and type firewall and select Windows Firewall.

Select Turn Windows Firewall on or off on the left-hand side. Then turn off all the Firewalls.

Here we are failing the posture check since we turned off the Windows Firewall. Click Cancel.

Hit Yes to cancel.

This process will run twice just go through the cancellation process again

Here we can see that we are not compliant. We should be switched to VLAN 35 and have internet only network access.

Here we see we got an IP from VLAN 35.

Internet is working but we can’t access internal resources except ISE.

ISE live logs show that we came in unknown then went to non-compliant. Check the details and see what information is there.

If we look at the switch we can see that we have the correct ACL and are being assigned to VLAN 35.

If we go back to PC-2 and click on the Gear Icon on the AnyConnect Client then go to System Scan take a look at the Security Products tab. We can see all the Security products installed on the device.

Now move over to the Scan Summary tab. Here we can see our 2 checks with the Firewall check failing.

We have one more thing that we configured, that was application collection. To see what we have collected in ISE go to Context Visibility > Endpoints and find ISE-PC-2 and click on its MAC address.

Now in the applications tab you should be seeing all the installed and running applications along with other information, version numbers and install path.

Now we’ve tested our unknown and not-compliant rules. Let’s test the compliant policy but instead of just turning on our Windows Firewall let’s have ISE do that for us.

In ISE go to Work Centers > Posture > Policy Elements > Remediations > Firewall and Edit ASTON-FW-WINDOWS. Change the Remediation Type to Automatic and hit Save.

Then go to Requirements and find the ASTON FW WINDOWS rule change the Remediation Actions to ASTON-FW-WINDOWS.

Log into the Access-SW and shut the port on G1/0/10 then disable the network adaptor on ISE-PC-2.

Then no shut the port and enable the network adaptor. Now you should come back online as compliant and if you check the Firewall you’ll see that it has been automatically enabled.

If we look at ISE we see that we get the EMPLOYEE COMPLIANT policy and the EMPLOYEE ACL.

And the switch:

Conclusion

In this lab, we have:

- Configured client provisioning

  - Created an AnyConnect Posture Profile

  - Uploaded an AnyConnect package

  - Created an AnyConnect configuration package

  - Configured client provisioning policy

  - Created a client provisioning portal

- Configured our Policy Elements for Posture

  - Created conditions for App collection, Windows Firewall and Windows Defender

  - Created dACLs for unknown and non-compliant policies

  - Created Authorization Profiles for compliant, non-compliant and unknown

- Configured our Posture Policy

  - Configured our WIRED Policy Set to support Posture

- Tested our posture policies for unknown and non-compliant

- Changed our FW policy and tested for compliant status

In the next lab, we are going to cover posture for guests utilizing the temporal web agent.